STOS ON THE STE:

:1 SOUND..........................Terence Pearson

:2 STE EXT........................Terence Pearson

:3 STE DAC........................Uknown Author

:4 ALL ABOUT THE STE AND STOS.1...Jens Hucke

:5 ALL ABOUT THE STE AND STOS 2...Jens Hucke

:2 HARDWARE SCROLLING.............Jens Hucke

STOS STE EXTENSION

~~~~~~~~~~~~~~~~~~~

Part 1 -  Sound Commands

~~~~~~~~~~~~~~~~~~~~~~~~

By Terence 'TEZ' Pearson

~~~~~~~~~~~~~~~~~~~~~~~~

Welcome to Part 1 of my STE EXTENSION guide. In this part we are going to

look at the commands offered by the extension.

Let's start with the commands that I'm most familiar with, the DMA sound

commands. All the sound commands begin with DAC, which stands for Digital

to Analogue Converter.

Using these commands you can play samples recorded using MASTERSOUND II,

STEREO MASTER and any other sampler that saves samples in signed format,

that is provided they were recorded at 6.25kHz, 12.5kHz, 25kHz or 50kHz.

DAC SPEED n

10 DAC SPEED 1

This sets the playback speed where n is a value from 0 to 3

0 -  6.25kHz

1 -  12.5kHz

2 -  25.0kHz

3 -  50.0kHz

DAC MONO

10 DAC MONO

This sets up the DMA hardware ready to play a MONO sample.

DAC STEREO

10 DAC STEREO

This sets up the DMA hardware ready to play a STEREO sample.

DAC CONVERT start address,end address

10 DAC CONVERT start(15),start(15)+length(15)

Using this command you can convert an unsigned sample to signed

format. Start and end are the start and end addresses of the sample

that you wish to convert.

DAC LOOP ON

DAC LOOP OFF

10 DAC LOOP ON

When a sample is played it will play continuously.

10 DAC LOOP OFF

When the sample is played it will only play through once and then

stop.

DAC RAW sample start address,sample end address

10 DAC RAW start(15),start(15)+length(15)

This command plays the sample between the two addresses start and

end.

DAC M VOLUME n

10 DAC M VOLUME 40

This command sets the overall volume of the sound output.

n is the volume and can be between 0 and 40.

DAC L VOLUME n

DAC R VOLUME n

10 DAC L VOLUME 15

20 DAC R VOLUME 18

These two commands allow independent volume control for the left

and right sound outputs. n is the volume and is a value between 0

and 20.

DAC STOP

10 DAC STOP

This commands stops any sample that is being played.

DAC MIX ON

DAC MIX OFF

10 DAC MIX ON

DAC MIX ON allows samples and sounds from the ST's PSG sound chip

all to be played through the DAC together.

STOS AND THE STE PART 2

~~~~~~~~~~~~~~~~~~~~~~~

E PALETTE $RGB,$RGB,$RGB,$RGB.......(16 colour values)

10 E PALETTE $F00,$E00,$D00,$C00,$707,$101,$211,$000,$46A,$5A5,

$414,$333,$555,$FFF,$BBB,$999

This is the same as the PALETTE command in STOS, and can be used to

define the colour palette.

RGB are red, green and blue values in hexadecimal. The values you

can use are 0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F

E COLOUR n,$RGB

10 E COLOUR 5,$7A0

This command is used to set one colour, again the RGB values are

hexadecimal. This is the same as the COLOUR command in STOS.

x=E COLOR(c)

10 x=E COLOR(5)

20 PRINT HEX$(x,3)

The command x=E COLOR(c) returns the RGB value of colour c into the

variable x.

x=STE

10 x=STE

20 IF x=1 THEN PRINT "This is an ATARI STE"

30 IF x=0 THEN PRINT "This is not and ATARI STE"

Using this command you can detect if your program is running on an

STE or not.

x=LIGHT X

y=LIGHT Y

10 REPEAT

20 x=LIGHT X

30 y=LIGHT Y

40 PLOT x,y

50 UNTIL FALSE

If you have a light pen or light gun attached to the analogue joystick

port of your STE then the x and y co-ordinates on screen can be returned

with these commands. LIGHT X and LIGHT Y contain the x and y co-ordinates

when the fire button or light pen button is pressed. See x=FSTICK(j)

x=LSTICK(j) : REM Left

x=RSTICK(j) : REM Right

x=DSTICK(j) : REM Down

x=USTICK(j) : REM Up

x=FSTICK(j) : REM Fire

These commands are used for detecting the joysticks, j which can be from

1 to 6.

x=FSTICK(3) can be used to detect the light pen/gun's fire button.

STICKS ON

STICKS OFF

This switches on interrupts for joystick ports one and two.  STICKS ON

also disables the mouse, and so if you need to use the mouse again then

you need to use STICKS OFF

x=STICK1 : REM Joystick in mouse port

x=STICK2 : REM Joystick port

These two commands can be used to return the status of joysticks one and

two.

Value of x returned :-

1 - Up

2 - Down

4 - Left

8 - Right

128 - Fire

For diagonals x will be the sum of the values, for example :-  If up and

left are pressed together x will be 5 If fire and right are pressed then

x will be 136

There are some hardware scrolling commands, but try as I may I can't

get them to work. If YOU know how they work please tell me!

Have fun with these commands, and maybe we'll see some more STOS STE

games!.

/��������������|��������������|��������������\    \���\

/               |              |               \    \   \

�������/   /�����|   |����������������������/   /     \   \

/   /      |   |                     /   /       \   \

/   /       |   �����������|         /   /         \   \

/   /        |              |        /   /           \   \

/   /         |   |����������        /   /             \   \

/   /          |   |                 /    |              ����

/   /           |   �����������|     |     �����������|   \����\

/   /            |              |     |                |    \    \

����              ��������������       ����������������      �����

STOS AND THE STE

****************

Ok, so your ST has got that  little  'E'  added to your sticker on the

cover of your machine but what does it mean as a STOS programmer?.

Well if you have got  the  STE  extension  you can access the hardware

scrolling routines,(If you can get   it  to  work that is!) access the

extra   joystick/light    gun/pen    ports     on     the    left   of

yourmachine, unfortunatly they are non  standard   ports joystick ports

(Nice  one  ATARI   give   us   analogue   ports    and   no  analogue

joysticks!!).  You can  also  gain  access  to  the   increased   colour

palette,  from 512 to 4096 colours,(But  you  can still only use 16 on

screen!!)  and  almost  finally   the   BLITTER,  wow  what  an  amazing

diffrence  in  speed!! NOT!  Im  pretty  sure  that  most  of the  TOP

NOTCH  copy routines in MISTY/Missing link  are faster. You can though

copy Graphics  to anywhere on  screen  not  in  a 16 pixel boundary.It

does run along side the main processor so takes zero processor time to

work it  but  it  still  needs  that  bit  extra...  What about it TOP

NOTCH??? Blitter sprites??

Lastly we come to the best part of the STE hardware......

The     D.A.C.   (Digital to Analogue Converter)

There are 2 ways to access  this  chip from STOS, the easy way

is to get hold of the STE EXTENSION as on ST FORMAT cover disk NO.

or from your P.D. Libary.

But fear not if you do  not  own  the above extension ( Or you

want to save a couple of quid) you can do it the long way...

(BOTH LISTINGS ARE ON THE DISK AS WORKING EXAMPLES)

1 rem      *****************************************************

2 rem                          STE DAC ROUTINE

3 rem      ****** As used in STORM by Fugitive Freelancers *****

4 rem      *****************************************************

5 if length(10)=0 then reserve as data 10,23634 : bload "sample.sam",10

6 rem

7 rem

10 DMA=$FF8901 : rem                  Address of DMA chip

11 rem

20 SAM=start(10)+90 : L=length(10)-99 : rem Start and finish of sample data

21 rem

30 poke DMA,0 : rem                   Reset DMA

31 rem

40 poke DMA+32,128 : rem              Set playback speed

50 rem                                128=6.5  129=12.5  130=25.6  131=51.3

51 rem

60 poke DMA+2,SAM/$10000 : rem        Set start and end of sample bank

70 poke DMA+4,SAM/$100

80 poke DMA+6,SAM and $FF

90 poke DMA+8,SAM/$10000

100 poke DMA+10,SAM/$100

110 poke DMA+12,SAM and $FF

120 poke DMA+14,(SAM+L)/$10000

130 poke DMA+16,(SAM+L)/$100

140 poke DMA+28,(SAM+L) and $FF

141 rem

150 poke DMA,3 : rem                  Play sample: 1-Normal play                                                                   3-Looped sample

0-Stop Sample

199 rem      *****************************************************

200 rem                     MICRO WIRE INTERFACE

201 rem            (ADJUST volume/treble/bass/left/right)

202 rem      *****************************************************

203 rem

210 doke $FF8924,$7FF : rem           Set up mask register.

211 rem

219 V=40 : rem                        SET MAIN VOLUME

220 doke $FF8922,%10011000000+V : rem V=0 to 40 (use 29 or more to hear it!!)

221 rem

229 B=6 : rem                         SET BASS

230 doke $FF8922,%10001000000+B : rem B=0 to 12

231 rem

239 T=10 : rem                         SET TREBLE

240 doke $FF8922,%10010000000+T : rem T=0 to 12

241 rem

249 L=20 : rem                        SET LEFT VOLUME

250 doke $FF8922,%10101000000+L : rem L=0 to 20

251 rem

259 R=20 : rem                        SET RIGHT VOLUME

260 doke $FF8922,%10100000000+R : rem R=0 to 20

And now for the Extension version...

10 rem      *****************************************************

20 rem                          STE DAC ROUTINE

30 rem      ****** As used in STORM by Fugitive Freelancers *****

40 rem      *****************************************************

50 if length(10)=0 then reserve as data 10,23634 :bload"sample.sam",10

60 rem

70 rem

80 dac speed 0 : rem                    set replay speed  0=6.25

1=12.5

2=25.6

3=51.3

81 rem

90 dac loop on : rem                    loop sample on/off

91 rem

100 dac mono : rem                      sample mono or stereo

101 rem

110 dac m volume 40 : rem               set main volume 0-40

111 rem

120 dac l volume 20 : rem               set left volume 0-20

121 rem

130 dac r volume 20 : rem               set right volume 0-20

131 rem

140 dac treble 12 : rem                 set treble 0-12

141 rem

150 dac bass 8 : rem                    set bass 0-12

151 rem

160 dac mix on : rem                    mix with normal ST sound on/off

161 rem

169 rem            Set start and end of sample data and play.

170 dac raw start(10)+90,start(10)+length(10)-99

180 rem                                 DAC STOP will halt sample

A bit shorter and clearer to understand me thinks..

Both of these routines once set up  and running take up zero processor

speed so all graphics/calculations ect do  not slow down. Just a short

looped sample like in the  example  can  sound  loads better than some

bleepy chip music.(In my opinion anyway!!)

Q.  But how do you use  loads  of  different samples in your new game?

You can't really use all 15 Banks to  hold a sample each so how do you

do it?

A.  When you are collecting your  samples together hold them one after

another and make a note  of  the  address  of  each of in your sampler

package. At the end you'll get a list somthing like this:-

0-14536                 Bass Hum-Hum

14536-19239             Door opening

19239-23958             Machine gun

23958-26499             Lazer gun

Then when you call the sample you want  (eg Machine Gun) all you do is

add the start and end  address  to  your  bank .no! Confused?? OK like

this......

Dac raw start(10)+19239,start(10)+23958

this will play the Machine Gun sample.

Got it?   GOOD.

**************************************

STOS AND THE STE

~~~~~~~~~~~~~~~~

A new column dealing with the STE side of STOS Programming.

By J.J.

BLITTER AND STE COMMAND TUTORIAL BASED ON ASA BURROW's DOC but severely

edited to make it more usable. Hope he doesn't mind to much. If you read

this Asa, please send any input you might have to me.

To follow these tutorials, you will need:

STOS

The BLITTER .EX*                Starting this month

The STE_EXTN.EX*                Starting soon

A Blitter chip, and/or an STE

Intro:

======

This new section in Stosser will deal with aspects of the STE such as the

Blitter, DAC and Hardware scrolling. Once again, many thanks for the

people writing the extensions, and in this case ASA Burrows. Having met

him, I can only say he is a very nice chap, and it is a shame that more

people are not making use of his extensions.

Here is the challenge: Can you use these extensions?

Have you a demo screen?

Have you got a clue? (I haven't!)

Can you help!

Appeal:

=======

Yes, I know! I haven't even started and already I am appealing for your

source code and routines. But hey! why not. Please, please, please send

me stuff STE related with plenty of Docs and Rems so that I know what is

going on! How many of you are actually using STEs out there?

*** Warning **********************************************************

*All the stuff I mention is mostly trial and error, so I can't offer *

*any guarantees. This tutorial might be harmful to your machine,     *

*or peripherals, so be warned!                                       *

*P.S.: I never had any problems, but who knows.                      *

**********************************************************************

Why bother with the STE and Blitter extensions?

===============================================

Many of you will be using the Missing Link extension and mighty it is

too! But let's face it, the best machine code can not outperform hardware

trickery. Imagine a Horizontal 320*200 screen scroll that hardly uses

any machine time at all! The possibility to shift large chunks of

graphics around without fancy programming and speed penalty. Better

sound. All elements that would allow you more freedom to do animation.

That is the theory at least, But have you noticed that so far no use has

been made by STOS users of these extensions? I could be wrong, in which

case please send me your code! I could be right, in which case I hope

this column will provoke some kind of reaction and motivation.

A quick summary follows, describing what can be done. An in depth

Tutorial will start afterwards.

First here is a summary of what is available to you:

In the Blitter Extension:

Macros: set of high level commands doing the equivalent of "screen copy,

cls, fill," but faster and easy to use.

Expert Commands: Access to all the registers of the blitter chip! Very

Very Fast but Difficult to use. Allows Single plane

manipulation! and logical operations.

Frighteningly fast !!!

The blitter chip can handle most common graphic tasks faster than the

68000, and leaves the main processor free to do other things, with a

minimum speed penalty. I love it for scrolling routines... that beat

anything else I have seen! (I managed to crash my ST over 100 times

writing this doc! so be careful and save everything)

In the STE Extension

Twin joystick support

Light Gun

Extended palette         * so far nothing new or

important

DAC  or DMA sample playing routine

Hardware scrolling       * the powerful and painful bit

Now let's start some tutorials:

PART 1       ******** THE BLITTER EXTENSION  *********

Here are the macro commands. They are the easiest part to master and the

tutorial finishes with an example covering most commands.

x=BLITTER

---------

This checks to see if a blitter is fitted. If a 1 is returned, you have

one, if 0 is returned you don't. So if you get a "0", enjoy the rest of

STOSSER and goodbye!

BLIT CLS screen

---------------

Clears the screen. This can be logical, physical, or the address of a

screen bank.

i.e.:

20 load" picture.neo",physic

30 wait key

40 BLIT CLS physic

*** not very clever !  loads a pic and erases it.

BLIT COPY source,destination,[op]

---------------------------------

BLIT COPY source,destination,op

BLIT COPY source,x1,y1,x2,y2,destination,x,y

BLIT COPY source,x1,y1,x2,y2,destination,x,y,op

This one is more useful! It's similar to Screen Copy in Stos, or Blit

from the Missing Link. So you should know how to use it!

If you are copying only a small part from the source address, remember

that the X coordinates will be rounded to the nearest 16 pixels. The good

news is that the destination can be any X,Y coordinates.

The optional op parameter can range from 1 to 14 and sets the type of

copying to do. Don't bother using it for a straight copy, but some nice

effects can be had

1  source AND destination

2  source AND NOT destination

3  source (a straight copy)

4  NOT source AND destination

5  destination

6  source XOR destination

7  source or destination

8  NOT source AND NOT destination

9  NOT source XOR destination

10 NOT destination

11 source OR NOT destination

12 NOT source

13 NOT source OR destination

14 NOT source OR NOT destination

(N.B.   XOR does a transparent copy)

i.e.:  we have our picture in bank 10 and want to copy the top left 1/4

to the centre of our physical screen.

...

...

50 blit copy 10,0,0,160,100,physic,80,50

...

Feel free to use different op parameters to see the effects.

REMEMBER THIS IS WHAT I CALL A MACRO. To squeeze more speed we will use

the low level expert (not!) commands at a later date.

X=BLIT BUSY

-----------

Very useful !

Remember that once you have instructed the blitter to do something it

will carry it out on its own! Normally  we use a "wait vbl" to make sure

all graphic operations are completed.

Using this command we can also check that the blitter has finished its

work before doing a screen swap. Omit this at your own risk

i.e. .......

200 wait vbl

210 if BLIT BUSY=1 then 210

220 screen swap

* simple or what!

********** WARNING **********

IF YOU STOP A PROGRAM BY USING <CONTROL <C>>

YOU ARE LIKELY TO CRASH YOUR STE.

NEVER STOP OR END A *.BAS WHILE THE BLITTER

IS ACTIVE !!

Ok, so far those of you using the Missing link might think that there is

no massive speed gain by using the blitter. Those used to standard STOS

should already be impressed. But hey, let's go really mad and use the

"expert commands." Those of you who know how the ST screen is built will

have a serious advantage.

The real McCoy:

===============

The following program loads a screen into bank 11, then blits it at a low

level to the logical screen, and displays it after a screen swap. It then

waits a while, and exits cleanly without a crash. Use this program to

mess about with different values, but always save your work first. No

automatic clipping is done, and as we are in supervisor mode, you are

likely to erase vital memory areas, causing a reset. As it is it doesn't

do much, but wait till next month!

10 mode 0 : flash off : logic=back           rem**************

12 reserve as screen 11                      line 10-16 setup

15 load "canyon.pi1",11                      for our screen.

16 get palette (11)

17 A=start(11)                                the variable "A"

contains the source

picture's address

1000 blit source address A                    We tell the blitter where

1005 blit dest address logic                  to copy from, and to.

1010 blit source x inc 2                     We copy all 4 planes at a

time,so "x inc 2".

If we wanted to copy a

single plane at a time it

would be "x inc 8". If we

wanted to copy every other

word "x inc 4"

The "inc" commands set the increment to the next word in bytes. I know

this will not make any sense to most of you, but I will you a better

explanation with precise examples next month!

1020 blit source y inc 2               the "y inc (n)" sets the

line increment in the

same fashion

1030 blit dest x inc 2                 As before but for the

destination

1040 blit dest y inc 2

1050 blit x count 80                   Sets how many words to

read per line.

1060 blit y count 200                  Sets how many lines

to blit over.

1070 blit endmask 1 $FFFF

1080 blit endmask 2 $FFFF              These 3 set the masks for

1090 blit endmask 3 $FFFF              the destination data.

The first mask is for the 1st word

The 2nd is for all the other words

The 3rd is for the last word

$FFFF copies everything

But you can use binary words:

%1111111111111111   total copy

%1010101010101010    stripy

or

%0000000000000000   destination

stays unchanged

or ... whatever you want!

1095 blit h line 0                     We could have defined a

halftone pattern earlier. If

we had, we would start using it

from line 0

Values for blit h line (n)

range from 0 to 15

1100 blit hop 2                        If we were using a halftone

pattern, we must tell the blitter

what to do with it. Value for

blit hop (n) are:

0 all ones

1 half tone

2 source

3 source and half tone

1110 blit skew 0                       This is the nice one! using skew

values from 0 to 15 in a step of

1, we can shift the destination

one pixel at a time to the right

or left by decreasing from 15 to 0

This only works when using it

on 1 plane at a time, or there

is a mess on screen

We will cover this in detail next month as it allows ultra smooth

horizontal scrolling .

1115 blit smudge 0                     If "blit smudge 1" then the value

from "skew is also used as the

"blit h line " number.

Here, it is set to 0 and ignored

1120  blit it                          We have set everything up,

so let's do it !

1130 if blit busy=1 then 1130          Wait for the blitter to finish

1999 screen swap:wait vbl              Screen swap to make the finished

work visible.

2000 wait 100 : cls physic : default : end

Waits a bit then exits nicely!

There are only a couple more  commands to cover:

Obviously, if you intend to use them, they must be placed before the

"BLIT IT " command, as they won't have an effect otherwise.

BLIT HOG

--------

Takes complete control of the STE to get extra speed. Stops most

interrupts, and please try to avoid it, unless you want to crash !

BLIT NFSR n

-----------

When n=1, the last source read on every line is not done (NO FINAL SOURCE

READ). (n=0, means normal)

BLIT FXSR n

-----------

If n=1, an extra source read is done on every line (FORCE EXTRA SOURCE

READ). (n=0, normal)

BLIT OP op

----------

This sets the logical operation of the data being copied. The operations

are the same as for BLIT HOP with two options:

0 all zeros

15 all ones

BLITHALFTONE pat1,pat2,,,,,,,,,,,,,,pat16

These enable you to set up a 16 x 16 half tone pattern. It's easiest to

enter the patterns as binary words,so each digit represents a pixel. As I

didn't set it up for my small program, I ignored it! Check lines 1095 and

1110 to see what it can be used for. If you find anything out on this

one, please send me the code.

THIS IS IT FOR THIS MONTH I AM AFRAID.

*********************************************************

NEXT MONTH:

-*- Screen structure explained

-*- In depth look at the Expert commands

-*- How to use the blitter to scroll the screen 320*200 in

16 Technicolor (?)

-*- Hyper Magnifier

-*- 1 bit plane parallax starfields

***********************************************************

PART TWO

~~~~~~~~

A column dealing with the STE side of STOS Programming.

By      J.J.

THIS MONTH: MORE ABOUT THE BLITTER

THE DAC Sound Interface

For the first part you will require:

An STE or blitter equipped ST

The BLITTER.EX*

For the Second part you will require

An STE or DAC equipped Atari

The STE.EX*

Many thanks for the people writing the Extensions, and in this

case ASA Burrows.

It is a shame that more people are not making use of

his extensions. Let's hope that this is changing now that

STOSSER is attacking it.

Appeal:

Yes, I know ! Again, Yawhn !!

Any source are not only welcome, but required. Frankly, if you

own an STE and are reading this, you know you have

a superior beast!

So use it!!!!!!!!!!!!!!!!!!!!!!!!!!!!

By the way, if anyone out there has some good

Alien Attack  wave generators, please send it in!

*** Warning **   ****************************************************

*All the stuff I mention is mostly trial and error, so I can't offer*

*any guarantees. This tutorial might be harmful to your machine,   *

*or peripherals, so be warned!                                      *

*ps: I never had any problems, but who knows.                       *

*********************************************************************

Hopefully you have enjoyed last months tutorial! Please don't swear

if you managed to crash your STE more often than ever before. The

Blitter commands are quite powerful, but not fool proof.

The learning curve is pretty steep, and without a Hard Drive, I never

would have been able to bear the constant reboots required to

get last months examples together.

Anyway, this month we will have some fun and get a fast horizontal

scrolling background going.

PART 2  of   ******** THE BLITTER EXTENSION  *********

I will go step by step, on the main issues, but included on the disk

are a couple of programs doing weird things.

We will also build a parallax scrolling stars background.

*****************************************************************

A small word about screens:      **** SIZE CONVENTIONS ****

We work in low resolution  320*200 pixels in 16 colours

In High res, each pixel is either on or off and represented by

a single binary digit called "bit"

In low res, each pixel requires 4 binary digits to get the proper

address of the colour register to us

0000    colour register 0

...

1111    colour register 15

The palette is what we call the 16 colour registers

8 bit= 1 byte     Why?   Because !

So in low resolution we have 200 lines of 320 pixels

1 pixel->4 bits

320    ->1280 bits=160 bytes

1 word= 2 bytes            Because!

320    ->80 words

So, we have 80 words of information per horizontal screen line.

Why do I bother telling you this? because last month

we had to use "words" in our EXPERT blitting routine

Flashback: (       BLIT X COUNT count

BLIT Y COUNT count

These 2 set the no. of words to be read on a line,

and the number of lines.       )

If we use

....

BLIT X COUNT 80

BLIT y COUNT 200

...

...

we should copy the entire screen, and this is exactly what happened.

*****************************************************************

Another small word about screens:        * BIT PLANES *

In low resolution, each 16 pixel group on the screen is represented by

four consecutive words in memory. The first word contains all the low

-colour bits for the group, the second word contains the next 16 bits

and so on...

so if we have                       w w w w

o o o o

r r r r

d d d d

1 2 3 4

Pixel 1         0 1 0 1

Pixel 2         1 0 0 1

Pixel 3         1 0 0 1

Pixel 4         0 1 1 0

Pixel 5         1 0 0 1

Pixel 6         1 0 0 1

Pixel 7         1 0 0 1

Pixel 8         0 1 1 0

Pixel 9         1 0 0 1

Pixel 10        1 0 0 1

Pixel 11        1 0 0 1

Pixel 12        0 1 1 0

Pixel 13        1 0 0 1

Pixel 14        1 0 0 1

Pixel 15        0 1 1 0

Pixel 16        1 0 0 1

word 1    0110111011101101        word 2    1001000100010010

word 3    0001000100010010        word 4    1110111011101101

The next 16 pixel groups are all encoded in the same way.

Guess what, you just learned why there is always a 16 pixel

limit to copy commands.

And you also just discovered why we talk about 4 bit planes graphics

in low resolution.

The first plane is in word 1,5,9....

The 2nd   plane            2,6,10,14,18....

The third plane            3,7,11,15...

The fourth  ...            4,8,12,16,...,16000.

note: 16000 words is 32000 Kbytes, total screen size

MAGIC!

Remember last months SKEW command?

At the time I said that we have to skew and blit one

bit plane at a time. It probably didn't make sense at

the time, but separating the planes will allow us to

avoid messing all the information up.

Try last months routine with different skew values

and check the mess out. This is what happens if you do

all 4 planes at the same time.

Here is the new code separating the planes

10 mode 0 : flash off : logic=back

12 reserve as screen 11

15 load "canyon.pi1",11

16 get palette (11)

17 A=start(11)

900 for sk=0 to 15                   rem     we will scroll 1 pixel

at a time to the next

16 pixel boundary

950 For pl=0 to 3                    rem     1 Bit-plane at  time

1000 blit source address A+pl*2      rem     offset between planes

1005 blit dest address logic+pl*2

1010 blit source x inc 8             rem     The blitter always  copies

1 word  at a time, and

will now jump to the next

word containing the same

plane, 8 bytes further

(or 4 words)

1020 blit source y inc 8

1030 blit dest x inc 8

1040 blit dest y inc 8

1050 blit x count 20                 rem     Only 20 words in total

4 planes*20 =80 words

1060 blit y count 200

1070 blit endmask 1 $FFFF

1080 blit endmask 2 $FFFF

1090 blit endmask 3 $FFFF

1095 blit h line 0

1100 blit hop 2

1110 blit skew sk                    rem     sk= 0 to 15

does the scroll

1115 blit smudge 0

1120  blit it

1130 if blit busy=1 then 1130

1140 next pl

1999 screen swap: wait vbl  :next sk         rem   display for 1 vbl

to stabilise

2000 wait 100 : cls physic : default : end

This should work! but as we have  large loops

it needs to be optimised. But then it is up to

you.

On disk you will find another program that scrolls over the

whole screen.

Note that there is no automatic clipping with the  blitter.

So it looks like the screen is wrapping around.

Data that disappears on the right of the screen, reappears

on the left, but 1 line lower.     Look at the *.Bas please!

Actually, this is down to the way the STs' memory is laid

out and isn't a problem.

Have fun finding your own clipping routines. Contributions

are welcome.

Also how about a routine that takes the screen

size, and plane number in consideration, and feeds the low level

routine all the required calculations automatically when copying

from sadd,x1,y1,x2,y2 to dest,x,y . This will be very useful

when we start using the Hardware scrolling.

Hardware Scrolling uses a window onto a large screen, so using

normal sprites  is out of the window (ha!):

Stos sprites get clipped when the coordinates are  out of 0,0,320,199

so when scrolling up or down ,the sprite will not been redrawn once we

have scrolled the first 200 lines.

Horizontal is even worse, as the sprites assume that every line  is 320

wide. If we widen the screen, they get distorted. If we double the screen

to 640*200, we get 1 line of sprite every 2nd line of screen, and again,

the clipping removes it altogether.

So if somebody can write a BLITTER BOB routine to do sprites, he

would be a hero. If it is any good, I am sure he'll get some

thing out of it. How about a bribe? Anybody corruptible out there?

Get in touch.          Wish list:

BODY= addr      (i.e.:logic) address for

Botter drawings

BOTTER n,x,y,img        Blitter bob

BOTCLIP x1,y1,x2,y2     Set clipping

x= BOTCLASH (n)         Collision test

BOB (n), returns

0 if no collision

or the number of

the encountered

sprite .

In the STOS_2.STE  folder you will also find  the Hyper magnifier

"ZOOMBLIT.BAS". Again it doesn't do much but to show what the chip can

do. The same picture is displayed as usual, but only the top part. The

Blitter is set up to copy the top half to the logical screen on every

second line . It is then run again,to fill in the blanks and this way

magnifies by 2 the original image. While this goes on, we also scroll the

screen.

**************************************************************

PARALLAX STARS

Lets' get lazy! The source is on the disk so please help yourselves. We

are using the fact that the blitter can copy 1 Bit plane at a time to

speed up the process.

The challenge to you is, to send me a modified .BAS file where the Stars

do not corrupt the background. There is nothing to be won, but if I get

sent something really good, it might find its way on to Stosser, and you

will be Famous among us ever after. And, I would be grateful for any

trouble you take to send us some feedback.

**************   ADVICE ON USING THE BLITTER  **********

BEFORE USING THE BLITTER LOW LEVEL COMMANDS

IN A PROGRAM, USE A MACRO TO EITHER COPY A

SCREEN AROUND OR TO CLEAR A SCREEN

THIS HELPS TO INITIALISE THE BLITTER CHIP

AND AVOID DISTORTIONS ON THE SCREEN

AT A LATER TIME.

SHOULD YOUR PROGRAM NOT WORK FOR AN UNKNOWN

REASON, SAVE YOUR PROGRAM, RESET THE MACHINE,

AND TRY AGAIN.

The blitter does sometimes get confused, and a machine reset will

initialise it again.

THIS IS IT FOR THE <<<<BLITTER>>>> I AM AFRAID.

At least for the moment!

*******************************************

*******************************************

Welcome to the:

***     ***     ****

*  *   *   *   *                           ///-\\\

*  *   *****   *                         <  0   0  >

*  *   *   *   *                              *

***    *   *    ****                         \_/

****  *****   *****

*        *     *

*****    *     *****

*    *     *

****     *     *****

*********************

*********************

One of the best bits in the STE is the Microwire interface, and the

DMA sound  or Digital to Analogue converter (DAC).

While the "DAC instruction set" has already been covered in

STOSSER,this time around there is a very flexible sample player included

for you on the disk.

The example will also show you how to fing automatically the length from

a sample, rather than reading its size from the desktop.

Full documented source is included.

Again, ASA Burrows has put full control of the DAC in your hands with

the STE extension. This doc is very similar to his own, and every thing

is so obvious I don't have much to add. But I did include a couple of

hints along the way.

It is all very easy, and the example .BAS file will show you what to do.

Commands available are:

DAC SPEED x  set the replay speed

0=6

1=12.5

2=25

3=50    Khz

If like me you have Master sound, you will notice that you can not sample

and then replay the same sample at the same speed using the DAC.

Fortunately, there are some sample convert programs in PD that can.

Or you could use Videomaster at 12 Khz as the Videomaster program

was on a Glossy cover disk and allows all kind of different sampling

cartridges to be used.

DAC RAW saddr,eaddr

Play the Raw sample from address start to end address

Once this command has been used, the sample will play

until it is stopped by calling  DAC STOP or until the

end of the sample is  reached.

DAC CONVERT saddr,eaddr

Convert Maestro Samples to Raw for use by DAC. Again

remember to do your sampling at a rate the DAC can

replay(6,12,25 khz).

Load your sample as usual, then use this command

before your main loop. Once converted, it will replay

perfectly.

DAC MONO

DAC STEREO

Set replay mode to mono or stereo. Usually mono.

If you use the Stereo with mono samples, you will

notice a change of replay speed, but no problems.

Just amend the replay speed accordingly to what

you use, and to what effect you want to achieve.

DAC LOOP ON (/OFF)

Set the sample to loop or to end after playing it once

The Microwire interface gives us more control over sound output:

DAC M VOLUME v

DAC L VOLUME v

DAC R VOLUME v

The Master volume can be set from 0 to 40

The Left and Right Channel from 0 to 12

(dac l , dac r refer to left right volume when in

Stereo)

DAC BASS    x

DAC TREBLE  x

Set the Bass and treble from 0 to 12 for maximum

effect.

DAC MIX ON (/off)    Allows you to mix both the DAC output and the

normal ST sound chip output . CAn be useful if

using chip music for a game, and playing DAC

samples for the effects.

DAC STOP             Stop the DAC

Remember that you can edit most values such as Volume, bass,.... while

the sample is playing, so you can fade a sample in by raising the volume

in a small loop. Or in stereo, it can be faded from left to right.

Quite easy don't you agree! SO GET CODING NOW !

Please send us anything you write, to either STOSSER or me if STE.EXT

related.

The STE EXTENSION has also  twin joystick controls, knows how to read the

side analogue joystick ports and a light pen. This has all already be

explained in Stosser, and I won't cover it because :

1 Nobody has the analogue controls for their STEs

2 Nobody has a light pen

3 There are already more than enough methods to use Twin joysticks

4 It doesn't really interest me anyway!

On the other side, we will cover the Hardware Scrolling routine in the

next couple of months, as this seems to have everybody baffled. <MMMM!>

Take care.

Jens Hucke

Welcome back to the STE !

This month we will introduce the hardware scrolling routine on your

favourite machine, the  :

******  *******  ******

*          *     *

******     *     ****

*     *     *                    """""

******           ******              (*)�(*)

*

\_/

It is assumed that you have read the previous two issues dealing with the

blitter chip, as this will be used to generate our screen data. If you

have not, then get them now from a friend or your friendly PDL.

You will require :

One  STE machine

STOS

The Blitter extension

The STE extension (REGISTERED VERSION!)

STOSSER

************************************************************************

AS ALWAYS:         !!  WARNING  !!

STOSSER and Myself can not be held responsible for anything

happening to your equipment,hardware or software.

All the following information has been tried and tested,

but you can't hold us responsible in case anything damaging

happens.         Use at your own risk

************************************************************************

Frankly, the above disclaimer should not worry you, but

I have to cover myself in case somebody in the STATES

reads STOSSER and blows their head off in frustration while

following the tutorial.

REGISTRATION:

There is a STE extension in the PD, but that one didn't

work with the stuff I wrote following the .DOC file.

So,I hope that you have the full registered copy, otherwise

you are on your own.

*********************************

* Hardware Scrolling explained  *

*********************************

The standard ST is quite fussy as to where its screen address is.

For that reason we normally use a fixed logical and physical address

and swap between them. All the background data is moved to do

some scrolling

On the STE, the screen address can be anywhere. This means

that we can move the screen address to the background data to

display, rather than the other way round.

Obviously, it is much faster to change a single hardware register

than to shift 32000k of image data.

-----------> the result is Ultra Fast Scrolling

Here is the glitch:

If we are going to change the screen address to a new one, we

must make sure that we have data (the picture) there to be

displayed

So we will create picture data larger than 320*200, but only

display a small part of it (320*200) at a time.

--*(*)*--

Large pictures for vertical scrolling are created by adding

several pictures together one after another

So, if you copy a picture to  start(15), and another

to start(15)+32000, we have a screen 2 pictures high

in bank 15.

320*400

For Horizontal pictures, we must create the data by sticking

several pictures together line by line.

In order to get a  screen that is 2 screens wide, we

must copy the first line of the first picture, followed

by the 1st line of the other picture. Then the 2nd line

of both pictures,and so on.

The example files show how to use the blitter for this!

Once we have created the data, we must tell the STE what the screen

size is that we have created, and which part of it to display.

This Method is called windowing onto a virtual screen.

x=HARD PHYSIC(screen address)

screen address should be the address of your

image data bank.

All it does is set the origin of the image

data it will window onto

500 x=hard physic(start(15))

the data is in bank 15

HARD SCREEN SIZE w,h,0

Tells the STE the size of the data we are

windowing over

w and h are width and height in pixels

(0 means 0 ? if you know please tell me)

Hard screen offset x,y

Tells the STE where to position the Window

over our data

HARD INTER ON (/off)

Start or stop the Hardware SCrolling interrupt

Once you have set everything up. just use HARD SCREEN OFFSET x,y to move

around over your screen.

If you are going to use STOS animation or graphic commands, be warned

that STOS clips screen data at 320*200.

So once your sprite has reached x or y coordinates superior to 320,200

they will be erased even if displayed in the middle of the monitor's

picture. If you want to move stuff around, bare this in mind and try to

use the Blitter for all your graphic copying.

Also, we have to manually create a logical screen

You will have to make sure that the physical and logical

screens are identical, with the same background

30 reserve as work 14,640000

40 reserve as work 15,640000

...

150 physic=start(14)

160 logic= start(15)

...

Loop

....

600 x=hard physic(logic):screen swap

....

End loop

...

or something like that.

Anyway, experiment. And if you want more source code in STOSSER

relating to this subject, write to us. I will do my best to comply.

There are not many commands, and to use them for something useful

is quite hard, so good luck.

Please send me anything you might have written with the help

of these STE series. Depending on your reaction we will go

into more detail with anything you may find difficult or interesting.

Should there be any mail, please include a stamp for the reply,

or a S.A.E.

Jens Hucke

F8/26 Hyde Park Grds

London W2 2LZ

UK

BYE!!!