AT&T Technologies                                    Document No. AP88-10DMOS

November, 1988

NETWORK ECHO CANCELATION

WITH THE

WE(R) DSP16 FAMILY OF DIGITAL SIGNAL PROCESSORS

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

echo16.doc    This document.

echo.s	      Echo canceller, Appendix B: Source Code, updated 10/12/90

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Due to RAM requirements the current echo.s cannot be assembled for the DSP16.

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1988 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

INDEX

~~~~~~

Introduction                         (included below)

The Echo Canceler: Basic Principles  (included below)

Algorithms for Echo Cancelation

The NLMS Algorithm With Tap Leakage

Residual Echo Suppression

Near-End Signal Detection

Hardware

Software

Initialization Tasks

Foreground Tasks

Background Tasks

Performance

PC Demonstration

Conclusions

Appendix A: An NLMS Algorithm Tutorial

Appendix B: Source Code

References                 (included below)

INTRODUCTION

~~~~~~~~~~~~~

This application note describes the implementation of a network echo

canceler using the WE DSP16 or DSP16A Digital Signal Processor [1].

The echo canceler is based on the leaky normalized least mean square

(NLMS) adaptive filter with double-precision coefficients. When sampling

at 8~kHz and using the DSP16 at 36~MHz, echo path lengths of up to 18~ms

can be canceled. If single-precision coefficients without tap leakage

are used, then echo path lengths of up to 29~ms can be canceled. Using

the DSP16A at 60~MHz, echo path lengths of up to 32~ms or 53~ms can be

canceled, depending on whether or not double-precision coefficients

with tap leakage are used.

The WE DSP16 Digital Signal Processor Demonstration Board (DEMO16),

described in a companion application note [2], is used as an evaluation

platform. DEMO16 provides for the interfacing of both the network- and

hybrid-side signals as well as an RS-232C port for the real-time display

and control of the coefficients of the adaptive filter.

First, a brief description of the network echo problem and how echo

cancelers are used to alleviate it is given [3]. Next, a description of

the algorithms that comprise the echo canceler are presented [4,5,6].

Then, the hardware and software implementation of a 16~ms echo canceler

is described and performance with respect to the CCITT recommendation

for echo cancelers is discussed [7]. Finally, a demonstration system

that uses a PC and the DEMO16 board is described.

THE ECHO CANCELER: BASIC PRINCIPLES

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Echoes in telecommunications networks primarily occur at interfaces

between two-wire and four-wire circuits. A two-wire circuit carries

signals in both directions on the same pair of wires.  Four-wire

circuits consist of two circuits, each carrying signals in only one

direction.

Two-wire circuits are found in a telephone company's local loop plant;

that is, those circuits that connect the telephone in a home or office

to the telephone company's local Central Office facility. The advantage

of two-wire circuits is that, at least as far as wire is concerned,

their cost is half that of four-wire circuits. The disadvantage is that

it is difficult to amplify signals in this type of circuit since

amplifiers are inherently unidirectional devices.

Since it is easy to amplify signals in four-wire circuits, they are

typically used in long-distance connections between Central Offices.

Two- and four-wire circuits are interfaced to each other by a device

called a hybrid circuit. The interface is depicted in Figure 1. On

the four-wire side the received signal, x(t), enters the hybrid and

the return signal, s(t), departs. On the two-wire side both x(t) and

the transmitted signal from the telephone, u(t), are present. The

return signal is composed of the transmitted signal as well as a

modified version of the received signal, s(t) = u(t) + H{x(t)}.

Ideally, the hybrid would attenuate the reflected received signal to

the extent that the return signal contains only the transmit signal.

Actually, the average transhybrid loss is 11~dB with a standard

deviation of 3~dB over the range of frequencies between 500~Hz and

2500~Hz [3]. This is the most critical range of frequencies with

respect to echoes.

REFERENCES

~~~~~~~~~~~

[1] WE(R) DSP16 and DSP16A Digital Signal Processor Information Manual,

November 1988, AT&T Technologies.

[2] WE(R) DSP16 Digital Signal Processor Demonstration Board Application

Note, November, 1988, AT&T Technologies.

[3] "Transmission Systems for Communications," Bell Telephone Laboratories,

Inc., Holmdel, NJ, 1982.

[4] D.L. Duttweiler, "A Twelve Channel Digital Echo Canceler," IEEE Trans.

on Communications, Vol-com-26, No.5, May 1978.

[5] S.J. Orfanidis, "Optimum Signal Processing",

Macmillan, New York, NY, 1985.

[6] B. Widrow and S.D. Stearns, "Adaptive Signal Processing",

Prentice-Hall, Inc., Englewood Cliffs, NJ, 1985.

[7] Recommendation G.165, "Echo Cancelers," CCITT, Geneva (1980).

[8] "T7500 PCM Codec With Filters," AT&T Communication Devices Data Book,

March 1988.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=