AT&T Technologies                                    Document No. AP89-007DMOS

June, 1989

DISK-DRIVE SERVO CONTROL

WITH THE

WE(R) DSP16/DSP16A DIGITAL SIGNAL PROCESSOR

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

servo16.doc  This document.

servo16.s    Appendix A. Program Listing 1. Adaptive Servo Controller

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

SUMMARY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1989 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

--------------------------------------------------------

CONTENTS

~~~~~~~~~

Introduction

Control Systems

Disk-Drive Control Systems

Adaptive Servo Control

Disk-Drive and Controller Interface

Controller and CPU Interface

Acknowledgments

References

Appendix

(NOTE: Figures 1 through 7 are not shown.)

INTRODUCTION

~~~~~~~~~~~~~

This application note describes an adaptive control algorithm as

implemented on a WE DSP16/DSP16A Digital Signal Processor.  The

algorithm makes use of an adaptive inverse modeling technique that

incorporates a 40-weight transversal filter whose coefficients are

updated according to the least-mean-square (LMS) algorithm.  The

DSP16/DSP16A is an ideal candidate for implementing such a control

algorithm.  This 16-bit fixed-point device can perform up to 40

million 16 x 16-bit multiply/accumulate operations per second

(generating full 32-bit products), making it the fastest single-chip

digital signal processor available [1].  The serial interface provides

for easy interfacing to analog-to-digital (A/D) and digital-to-analog

(D/A) converters with little or no need for glue logic.  The parallel

I/O unit can interface to a 16-bit bus containing other microprocessors

or microprocessor peripherals.  The adaptive control code executes in

455 instruction cycles (11.37 micro-secs on a 25 ns device).

The application note begins with a brief overview of control systems,

followed by a section that discusses disk-drive controllers and, in

particular, those features of a digital signal processor that are

required to implement this type of controller.  The subsequent section

describes the specific adaptive control algorithm implemented by the

DSP16/DSP16A.  The last two sections address the interface between the

controller and CPU, and the interface between the disk drive and controller.

CONTROL SYSTEMS

~~~~~~~~~~~~~~~~

A control system (or controller) regulates a process (or plant) to achieve

a desired response from the process.  The three main components that make

up a control system are the transducers, the actuators, and the controller.

Transducers measure the behavior (or response) of the process (e.g., shaft

encoders and tachometers).  Actuators (such as motors) make up the plant.

The controller generates the appropriate commands for the actuator.

A system may, in general, vary in complexity from a simple closed-loop

feedback configuration to one that incorporates a number of compensators

(controllers) arranged in either a series or parallel fashion.  Designing

a control system corresponds to optimizing performance with respect to

system parameters.  Parameters that indicate performance include

steady-state error, transient response, relative stability, and

sensitivity to changes in these parameters [2].

The controller may be realized via analog techniques or with a digital

signal processor.  Digital controllers offer a distinct advantage over

their analog counterparts in that they provide for vastly improved system

performance.  Another advantage of using a digital-computer-controlled

system is that the control algorithm can easily be changed by changing

the program of the controller, especially if a programmable processor

(i.e., DSP) implements the controller.  Also, digital controllers

are insensitive to variations in temperature, humidity, and other

conditions that can affect component values in analog systems.

Moreover, environmental variations generally result in the thermal

expansion of head support arms, disks, carriages, spindles, bearings,

etc., thereby making an adaptive control scheme necessary.

DISK-DRIVE CONTROL SYSTEMS

~~~~~~~~~~~~~~~~~~~~~~~~~~~

In particular, this application note describes a disk-drive controller.

Figure 1 shows a block diagram of a disk servo control system.  By

definition, the process to be controlled corresponds to the drive input

of a servo motor that determines the position of the read/write head on

the disk.  The controller issues the appropriate commands to the servo

motor via the D/A converter.  The servo motor, in turn, moves the

read/write head from a given position to any desired track on the disk.

The feedback loop requires head positioning information as the head

traverses the disk surface or attempts to follow the desired data track.

One method of providing such position feedback is to permanently write

the information in the track between the data sectors.  This method

assigns a fixed number of sectors to each track.  The beginning of each

sector is defined by a magnetically prewritten identification field

that contains the sector address plus cylinder and head information.

Since sectors that are equally spaced around the disk provide this

information, position feedback is available only at discrete instants

of time as the rotating sectors pass under the head.  Thus, the

compensation loop/controller must operate as a sampled data servo system.

As such, it determines the desired read/write head position and the

position error between the desired position and the position feedback.

The design objective is to keep the position error minimized at all times.

A digital controller incorporates a filtering operation in its feedback

loop, minimizing the position error and using the position error to

control a motor via a D/A converter.  Simultaneously, the processor

must also maintain the disk drive speed at the desired level.  To do so,

the processor monitors a digital signal from a counter circuit that

indicates the speed of the spindle motor and issues appropriate control

commands to the spindle motor through a D/A converter.

Past and recent trends have indicated a consistent increase in the

demand for larger disk storage capacities.  Increasing the number of

tracks on a disk, while at the same time decreasing their width,

represents one method to meet this demand [3].  With this approach,

however, accurate read/write head positioning and tracking become more

difficult with increasing disk capacity, thereby rendering conventional

design approaches inadequate.  The demand for increased accuracy in head

positioning and tracking requires a more frequent sampling of the head

position than would otherwise be needed.  Moreover, this sampling rate

must increase as the time constant of the process to be controlled

decreases.  Therefore, the disk controller must be capable of high

sampling rates in addition to performing the math-intensive algorithms

inherent in digital control applications.  The required sampling

frequency is governed by the desired transient response characteristics

of the system (i.e., the closed-loop bandwidth).  A general rule suggests

a sampling rate that is ten times the system bandwidth.  The processor

chosen to implement the controller must have the speed and wordlength

necessary to achieve this sampling rate and properly implement the

desired control algorithm.  Maintaining computing accuracy for higher

sampling rates implies the need for processors with larger wordlengths,

since all of the changing information between successive samples may be

concentrated in the least significant bits of the word.

The DSP16/DSP16A Digital Signal Processor is an ideal candidate for

implementing such a controller.  This 16-bit fixed-point device can

perform up to 40 million 16 x 16-bit multiply/accumulate operations

per second (generating full 32-bit products), easily making it the

fastest single-chip digital signal processor available.  The serial

interface provides for easy interfacing to A/D and D/A converters with

little or no need for glue logic.  The parallel I/O unit can interface

to a 16-bit bus containing other microprocessors or microprocessor

peripherals.

ADAPTIVE SERVO CONTROL

~~~~~~~~~~~~~~~~~~~~~~~

Conventional design approaches give way to adaptive control schemes when

higher track densities demand increasingly accurate head positioning in

the presence of environmental variations.  Figure 2 illustrates one such

scheme [4].  According to this scheme, the servo plant output, c(n), must

follow a reference (desired) model output, s(n).  Those elements comprising

the reference model appear in Figure 1.  A digital controller, D(z), and

servo plant, G_p(z), comprise the reference model, R(z), while a servo

motor, G(s), and D/A converter comprise the servo plant.  The digital

controller itself may be designed using conventional methods.  Referring

again to Figure 2, the adaptive reference inverse model is an inverse

model of the servo plant which, when combined with the servo plant and

a reference model as shown in Figure 3, gives an output, y(n), that follows

the reference model output, s(n).  The adaptive reference inverse model is

pre-computed offline (explained later in this section).  Once an adaptive

reference inverse model is obtained, it is incorporated into the control

system as shown in Figure 2.  The servo plant is driven by the output

obtained from a copy of the adaptive reference inverse model.

The parameters that define the adaptive reference inverse model are

updated after each seek operation.  This ensures that the servo plant

follows the same profile as the reference model at all times.

In the following section, the equations that describe the servo plant,

reference model, and the adaptive reference inverse model are defined.

To begin with, we assume that the servo motor transfer function is given by

1

1)      G(s) = ---------

s(s+1)

A general rule suggests that the sampling frequency can be chosen as one-tenth

of the fastest time constant.  Since the time constant of the pole at s = -1

is 1 second, the time period, T, is chosen as 0.1 second.

Then, referring to Figure 1, the transfer function of the servo plant is

1-e**(-Ts)     1

2)      G_p(s) = [zero order hold][G(s)] =  ----------- ---------

s        s(s+1)

C(z)          / 1-e**(-Ts) \       z-1      /     1     \

3)      G_p(s) =  ------  =  ZT | ------------ |  =  ----- ZT | ----------- |

Y(z)          \  s**2(s+1) /        z       \ s**2(s+1) /

or,

0.004837z + 0.004678

4)      G_p(z) = ------------------------

z**2 - 1.9z + 0.9094

where ZT means "the z-transform of."

To achieve certain system characteristics, a digital controller (or

compensator) needs to be designed.  Several techniques exist for the

design of digital compensators.  The tradeoffs involved in designing

with different methods are explained here. With the frequency-response

procedures (Bode plot), an attempt is made to reshape the system

open-loop frequency response to achieve certain stability margins,

transient response characteristics, steady-state response character-

istics, etc.  Thus the controller is designed with analog techniques,

and a digital approximation is obtained with the standard Z-transform,

the bilinear Z-transform, or the matched Z-transform.  A different

design is obtained by the use of root locus.  The root locus for a

system is a plot of the roots of the system's characteristic equation

as the gain is varied.  The design procedure is to add poles and zeros

via a digital controller to shift the roots of the characteristic

equation to more appropriate locations in the z plane.  Both the

phase-lag and phase-lead compensators can be designed.  The phase-lag

compensator allows a higher open-loop gain and a more stable system

for the same open-loop gain.  The phase-lead controller results in a

system with a smaller time constant; thus the system responds faster

(larger bandwidth).  As the conventional root locus diagram allows

only one parameter to vary at a given time, the design of a digital

controller in the z plane using a root locus diagram is essentially

a trial-and-error method.

For systems with more than one signal in the feedback path,

pole-assignment and state estimation techniques can be used.

The design results in the assignment of the poles of the closed

loop transfer function (zeros of the characteristic equation) to

any desired locations.  This technique requires the measurement of

all the states of the system if possible.  If not, some states of

the plant could be estimated from the information that is available

concerning the plant.  This results in the design of an observer or

a state estimator.  In pole-assignment technique, it is assumed that

the pole locations that yield the best control system are known.

However, another optimal design technique to obtain the best control

system is to write a mathematical function called the cost function.

The design procedure minimizes the cost function, resulting in an

optimal control system.

In this example, a phase-lead controller is designed with root locus

techniques.  The transfer function of the controller is given by

3.15(z**2  - 1.9z + 0.9094)

5)   D(z) =  -----------------------------

z**2  - 1.6847z + 0.7147

Now, the reference model transfer function is given as

6)   R(z) = D(z)G_p(z) =

/ 3.15(z**2  - 1.9z + 0.9094) \   / 0.004837z + 0.004678 \

| ----------------------------- | | ---------------------- |

\  z**2  - 1.6847z + 0.7147   /   \ z**2  - 1.9z +0.9094 /

or,

C(z)        0.01524z + 0.0147

7)   R(z) = ------ = ---------------------------

E(z)     z**2  - 1.68476z + 0.7147

This reference model may now be used to derive the adaptive reference

inverse model of the servo plant.  Figure 3 illustrates the adaptive

reference inverse modeling technique. This particular model incorporates

a 40-weight transversal filter whose coefficients are updated according to

the least-mean-square (LMS) algorithm.  The compromise between accuracy

and computational complexity dictates the choice of the number of taps in

the transversal filter.  The following equations describe this model:

servo plant output:

c(n) = 0.0048*x(n-1) + 0.0046*x(n-2) + 1.9*c(n-1) - 0.9094*c(n-2)

reference model output:

s(n) = 0.01524*x(n-1) +0.0147*x(n-2) + 1.68476*s(n-1) - 0.7147*s(n-2)

adaptive reference inverse model output:

i=N

y(n) = SUM w(i)*c(n-i+1);     N = 40

i=0

error:  e(n) = s(n) - y(n)

weight vector update:  W_i = W_i + u*e*c(n-i+1);     i = 1, 40

Taking the inverse z transform of G_p(z) and R(z), as given by equations (4)

and (7), respectively, yields the difference equations for the servo-plant

output, c(n), and the reference model output, s(n).  The weight vector,

W_i, which represents the adaptive reference inverse model, is obtained

by performing 2000 iterations of the adaptive loop.  The parameter u,

which determines the rate of convergence for obtaining the weight vector,

is chosen empirically, in this case, to be 0.05.  The input to the system

is assumed to be a step.  Once the adaptive reference inverse model is

obtained, it can be applied in the control system of Figure 2.  Due to the

adaptive nature of the inverse model, any variations in the internal

variables of the servo plant result in corresponding changes in the

coefficients of the adaptive reference inverse model.  Hence, the servo

plant output follows the reference model at all times.  The following

equations describe the control system of Figure 2.

servo plant output:

c(n) = 0.0048*u(n-1) + 0.0046*u(n-2) + 1.9*c(n-1) - 0.9094*c(n-2)

reference model output:

s(n) = 0.01524*u(n-1) +0.0147*u(n-2) + 1.68476*s(n-1) - 0.7147*s(n-2)

adaptive reference inverse model output:

i=N

y(n) = SUM w(i)*c(n-i+1);     N = 40

i=0

servo plant input:

i=N

u(n) = SUM w(i)*x(n-i+1);     N = 40

i=0

error:  e(n) = s(n) - y(n)

weight update:  W_i  = W_i  + u*e*c(n-i+1);     i = 1, 40

The DSP16/DSP16A adaptive control code, which appears in Appendix A,

executes in 455 instruction cycles (11.37 us on a 25 ns device).

Figure 4 shows the step response of the uncompensated system, while

Figure 5 shows the step responses of the reference plant and the servo

plant after adaptive compensation.  These figures clearly show how

closely the servo plant output follows the reference plant output.

In the case of a disk drive system, the compensated system would allow

much faster and accurate head positioning for read/write operations.

DISK-DRIVE AND CONTROLLER INTERFACE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Before reading and writing information on the storage medium, several

disk operations must occur.  First, the read or write location must be

defined and the read/write head must be moved to the desired track.

This operation is called a seek.  Next, the analog signal representing

the information must be digitized and applied to or extracted from the

disk.  Figure 6 shows the hardware elements that effect these operations.

Most disk drives have multiple platters for data storage.  In such

devices, track 0 of the first platter sits over track 0 of the second,

third, and remaining platters.  Thus, particular tracks on all platters

form spatial cylinders, so that selecting a track corresponds to

positioning the R/W head over that cylinder.  Specifying one of these

multiple platters requires the specification of the head address.

Commands that select a specific head in the cylinder enter the drive

via the controller (DSP16/DSP16A).  Those commands that specify a given

head cause the DSP to route selection signals over to the parallel bus.

Once a particular head has been selected, control signals are sent

through the D/A to the servo motor.  The demodulated data from R/W

head is read into the serial port through an A/D converter.  At this

point, the DSP16/DSP16A computes the filter and sends the appropriate

control signals to the servo motor.  Upon completion of the seek mode,

the data is transferred from the disk to dual-port RAM. The converters

used in this example are PCM78 (A/D) and PCM56P (D/A), which are

16-bit serial converters.

CONTROLLER AND CPU INTERFACE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The need for a controller to have the capability to accommodate multiple

host computer buses and attach a variety of storage devices led to the

introduction of standard interfaces.  An example of this is the Small

Computers Systems Interface (SCSI).  With this high-level interface for

host computers, certain types of storage devices (disk, tape, floppy,

optical disk, etc.) can be interchanged freely without requiring

modification to the host system hardware or software.  The attachment

of this interface to a particular CPU bus requires a host adapter.

Figure 7 shows the block diagram of the controller/host CPU interface.

The data stored on the disk is encoded in one of several popular formats,

including modified frequency-modulated (MFM) form, etc.  The data

separator decodes the data on the disk into binary information that

the CPU can understand.  The sequencer block performs disk formatting

and data read/write operations, as well as error correction and

serialization/deserialization of the data from the host interface.

The dual port buffer allows for differences in speed between the host

bus and the disk.

REFERENCES

~~~~~~~~~~~

1. WE(R) DSP16 and DSP16A Digital Signal Processor Information Manual,

Jan 1989, AT&T Microelectronics, 555 Union Blvd., Allentown, PA  18103,

1-800-372-2447.

2. Charles L. Phillips and H. Troy Nagle, Jr., Digital Control System

Analysis and Design, Prentice-Hall, 1984.

3. Jonah McLeod, Winchester Disks in Microcomputer Systems,

Elsevier International Bulletins, 1986.

4. Bernard Widrow and Samuel D. Stearns, Adaptive Signal Processing,

Prentice-Hall, 1985.

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=