AT&T Technologies                                    Document No. AP89-008DMOS

June, 1989

LINEAR PREDICTION BASED DTMF DETECTION

FOR THE

WE(R) DSP32 DIGITAL SIGNAL PROCESSOR FAMILY

NOTICE

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Copyright (c) 1989 AT&T -- All Rights Reserved

--------------------------------------------------------

WE is a registered trademark of AT&T.  AT&T reserves the

right to make  changes  to  the product(s), software, or

circuit(s) described herein without notice. No liability

is assumed as a result of their use or  application.  No

rights under  any patent  accompany the sale of any such

product or circuit.

DISTRIBUTION FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

dtmf32.doc	This document.

madtmf3.s	Program Listing 1. Linear Prediction Based DTMF Receiver.

madtmf2.s	Program Listing 1. Version 2.

OTHER PROGRAMS:

sadtmf12.s	Implements 1 DTMF receiver.

sadtmf12.ex	Simulator command file for sadtmd12.s

sis1.d		Data input file for sadtmd12.s

sadtmf22.s	The same as sadtmf12.s except valid digits are written to

the lower 8 bits of pdr instead of to RAM.

USAGE NOTES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-===-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

This is version 2 of the adaptive dtmf receiver.  It contains the aperture

adjustment that was erroneously omitted in version 1 and some fine tuning of

the narrow angle window.

This README is designed to take you from simulating 1 DTMF receiver to testing

32 DTMF receivers in real time.  The floppy contains 3 DTMF receiver programs

(sadtmf12.s, sadtmf22.s, and madtmf2.s) which will be described below.

sadtmf12

~~~~~~~~~

sadtmf12.s implements 1 DTMF receiver.  When a valid digit is detected it is

written to the integer RAM location called output.  All succeeding digits are

appended to this location.  RAM is initialized to store a maximum of 100 valid

digits.  The assembled code (sadtmf12) can be run and tested in either the

simulator or the development system.

To test sadtmf12 in the simulator you need to create an external input file

of 8 bit mu-law samples of touchtone signals.  Since data is read into the

DSP32 through the serial input port you also have to set up the serial input

clocks (pild and pick).  To get started the files sis1.d and sadtmf12.ex

are provided to take you through a sample simulation. sis1.d contains 4 50ms

touchtone digits (1,5,9,13) each separated by 50ms of silence.  sadtmf12.ex

contains all of the commands needed to set up the serial input clocks,

identify the input file, set up a breakpoint and display the locations in RAM

that contain the digits (1,5,9,13) when they have been validated.  Before

executing this sample simulation be aware that the input file contains 2900

samples, therefore, it will take from 1 to 15 minutes to complete the

simulation and detect and then display the RAM locations containing the

valid digits (depending on the speed of your computer).  To run this sample

simulation you must have sadtmf12, sadtmf12.ex, and sis1.d in your present

directory and execute the following command:

d3sim -e -m3 sadtmf12

After approximately 691,582 machine cycles, your output will appear as:

output[0:6]=

0x0000: 1 5 9 13

The file sadtmf12.ex sets a breakpoint at pause which will be hit after 25

frames (2900 samples) have been received.  sadtmf12 can also be executed on

the development system by typing the following commands:

d3sim -d1 -m3 sadtmf12

ioc=0x2547

autowait=0

run

Now while the program is running you can interactively read RAM starting at

output to see if you have detected any digits in real time.

sadtmf22

~~~~~~~~~

sadtmf22.s is the same as sadtmf12.s except that it writes valid digits to

the lower 8 bits of pdr instead of to RAM.  When a digit is written to pdr

the pdf flag goes high. When pdr is read, pdf goes low. To execute sadtmf22

on the development system, type the following:

d3sim -d1 -m3 sadtmf22

ioc=0x2547

autowait=0

run

madtmf2

~~~~~~~~

madtmf2.s implements 32 DTMF receivers on 1 25MHz DSP32.  When a valid digit

is detected it is written to the lower byte of the pdr.  The lower 4 bits

contain the digit detected and the upper 4 bits contain the receiver number.

At worst case you have 200 usec to read the pdr before it is overwritten by

another receiver that has detected a digit.  DMA is used to control the

serial input.  Therefore, to test madtmf2 you must have up to 32 external

codecs and some external hardware that creates ick, ild, and sync.

madtmf2.s was designed by optimizing sadtmf22.s for 1 channel and then adding

the extra control instructions to make it 32 channels.  Therefore, it may be

useful to refer to sadtmf22.s to understand the program logic. If any changes

are made to the existing program logic, it must be assembled as follows:

d3make -m0 madtmf2.s

All of this information is explained in detail in the enclosed application

note, "An efficient DTMF Receiver for the AT&T DSP32".

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-the-end-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=